📌 Meeting Summary – Flatwire Machine & Process Requirements

Date: March 5, 2026
Duration: ~3 hrs 20 mins
Purpose:
Define end-to-end process, system requirements, and shopfloor workflows for the new Flat Wire Rolling Line, including receiving, production, traceability, SPC, pass schedules, edge conditioning, and ERP/PLC integration.

👥 Key Attendees (as referenced in discussion)

- Jaspreet Singh – Requirements & system flow coordination

- Tim – Process / production clarification

- Shannon – Manufacturing & operational workflow

- John – Operations / historical process knowledge

- Ralph – Machine/process capability & calculations

- Chuck – Planning & MPS

- Laureen – System/process alignment

- Bob – Slitting & edge conditioning workflow

- Mark / Bill – Pass schedule ownership & setup
(Additional participants referenced during discussion)

🧠 Key Discussion Areas & Decisions

1. Flatwire Line – Machine & Process Overview

- System converts aluminum rod → flat wire using:

  - Payoffs (dual position)

  - Wire drawing blocks (Die 1 & Die 2)

  - 12” flat mill

  - Optional 8” mill

  - 3-stand / 6” finishing mills

  - Gauge stands before dancers

- Hybrid and non-hybrid operation modes supported.

- Speed constrained by final take-up (~1800–2000 FPM).

- Dies primarily change only due to wear or product change.

2. Pass Schedule

- No auto-generation initially.

- Pass schedule database required, not a generator.

- Pass schedule defines:

  - Which components are active/bypassed

  - Die sizes

  - Hybrid vs standalone route

- Maintained manually by Maintenance / Engineering (like current Early Screen).

- Pass schedule to be applied after planning, not during item creation.

3. Receiving & Validation (Rod Material)

- Receiving via PO or EDI.

- Mandatory checks:

  - Gross weight (scale)

  - Diameter (measured or from PO)

- Validation rules:

  - Weight tolerance vs PO/EDI

  - Diameter tolerance vs PO

- Failure → Material suspended

- Chemistry documentation required; missing → suspension.

4. Traceability & Identification

- Rod requires unique identification, similar to coil logic.

- Discussion on Rod Code strategy:

  - Reuse coil numbering format with differentiation

  - Possible suffix/prefix (e.g., “R”)

  - Archiving historical ranges if needed

- Full traceability required through:

  - Welded rods

  - Mixed lot scenarios

  - Weight-based allocation when splitting

5. SPC & Quality Checks

- SPC checkpoints:

  - Incoming rod diameter (pre-check-in)

  - Post wire-draw diameter (after die changes)

  - Automatic gauge readings after:

    - 12” mill

    - Final 6” mill

- Manual SPC mainly at initial setup or die change.

- Automatic gauges used during run (set-and-forget approach).

- Sampling rules vary by customer & process stage.

6. Production Flow & Check-in

- Support dual payoff positions with welding between coils.

- Pre-check-in and check-in simplified:

  - Pre-check-in: Diameter + rod number

  - Check-in: Die info, pass schedule acknowledgment

- PLC updated after pass schedule acknowledgment.

7. Edge Conditioning & Slitting

- Edge conditioning performed cut-by-cut.

- Cuts may be temporarily staged on skids.

- System must:

  - Track each cut independently

  - Allow out-of-order processing

  - Re-sequence cuts logically for downstream steps

- SPC, buildup, and scrap captured per cut.

- Edge-conditioned status tracked until all cuts completed.

8. Labeling & Packaging

- Labels printed at spool/take-up including:

  - Alloy, width, gauge, temper

  - Gross/net weight

  - Rod / coil references

- Final output typically ≤1000 lbs per spool.

- Packaging flows directly to packing line (no intermediate conveyors initially).

✅ Action Items

🔧 Engineering / Manufacturing

- Define and maintain Pass Schedule Database structure

- Confirm die master list for validation

- Finalize SPC triggers (startup vs periodic)

💻 IT / Systems

- Implement:

  - Rod receiving flow

  - Pass schedule acknowledgment screen

  - Rod/coil traceability logic (including weld handling)

- Support:

  - Edge conditioning transactions (cut-level)

  - Web log entries per cut

  - Gauge data storage integration

- Define rod numbering strategy & archival plan

📊 Planning / MPS

- Introduce “ROD” source option alongside reroll/hybrid

- Ensure planning selects rod-eligible material correctly

🧪 Quality

- Finalize:

  - SPC sample rules per operation

  - Customer vs QC sampling logic

  - Handling of mixed lots & welded material